05/19/2022 – Thursday – FPGA-VHDL-Demonstration using ISE Version 12.2

FPGA = Field Programmable Gate Array

VHDL = Very High-Speed Integrated Circuit Hardware Description Language

The purpose of this paper is to demonstrate how to program the XILINX Spartan-3E FPGA in VHDL.

This is a 208 pin device. The ISE version 12.2 application is used. This demonstration is performed on the “ham05” PC using Windows 7 Pro. ISIM is also used to simulate the VHDL code. The object is to find a low cost way to learn how to program a FPGA in VHDL.

Spartan-3E FPGA

XILINX SPARTAN

XC3S500E

PQG208DGQ1749

This FPGA has 500K system Gates

Platform Flash In-System Programmable Configuration PROMs.

XCF04SV6

78152

475VS

This one has 4Mbits.

Programmer – Platform Cable USB

Model DLC9G

The hardware was purchased from WAVESHARE as shown below. “Open3S500E Package B”

Project Description

A device will be created using the FPGA to generate pulse waveforms that can be measured on an oscilloscope. The following six period waveforms shall be provided which have a 50% duty cycle:

- 10,000 Hz pulses

- 1,000 Hz pulses

- 100 Hz pulses

- 10 Hz pulses

- 1 Hz pulses

- 0.1 Hz pulses

The four LEDs shall be assigned to show the following pulses:

- Led01 = 100 Hz pulses

- Led02 = 10 Hz pulses

- Led03 = 1 Hz pulses

- Led04 = 0.1 Hz pulses

The main clock runs at 50Mhz. The 10,000 Hz signal is generated by taking the 50MHz signal and counting its pulses. A countdown counter is used which is initially set to 4999. When the counter is less than 2500, the signal is set to ‘1’. When the counter is greater or equal to 2500, the signal is set to ‘0’. A 10,000 Hz clock signal is generated from here.

The 10,000 Hz clock signal is used to generate the 1,000 Hz signal. A countdown counter is used which is initially set to 9. When the counter is less than 5, the signal is set to ‘1’. When the counter is greater or equal to 5, the signal is set to ‘0’. A 1,000 Hz clock signal is generated here.

Likewise, the 100 Hz, 10 Hz, 1 Hz and 0.1 Hz signals are generated in a similar way.

A YouTube Video was made of this to document the process of programing VHDL into the FPGA.

https://www.youtube.com/watch?v=zFG3abGHhgI

For additional background notes see:

“220519-FPGA-VHDL-Demo-Background-notes.pdf”