07/13/2022 – Wednesday – NUCLEO-L476RG-RPM-Control

This paper will discuss project “NUCLEO-L476RG-RPM-Control” which is hosted on the “WIN710 PC”. The IDE “CubeIDE 1.10.0” was used. The NUCLEO-L476RG has an 80MHz Clock. The Frame time was set to 400 Hz by using the TIM3 interrupt. The motor used is labeled Hawking Aerodrive Outrunner Brushless BC-3548/4 1100kv. The ESC is labeled TurBORIX ESC/30A. It is model FM30A. The RPM sensor is labeled “Hobby Wing” Brushless RPM Sensor.

Servo Simulator to NUCLEO-L476RG

| Servo Simulator | NUCLEO-L476RG |
| --- | --- |
| IN+ | ESC 5V |
| IN- | ESC GND |
| OUT S | PB14 – TIM15_CH1 |

Channel 1 = Input Capture Direct Mode

Channel 2 = Input Capture Indirect Mode

PSC = 79

“Timer 15” is used. The pulse is scaled for 1E-6 sec/count. It has a normal range of 1ms to 2ms which translates to 1000 to 2000 counts. This can be used to drive the ESC directly or used as an input to the NUCLEO-L476RG, so the NUCLEO-L476RG can drive the ESC.

ESC to NUCLEO-L476RG

| ESC | NUCLEO-L476RG |
| --- | --- |
| GND | GND |
| SIG | PB6 – TIM4_CH1 |

Channel 1 = PWM Generation CH1 = PB6 = TIM4_CH1

Channel 2 = PWM Generation CH2 = PB7 = TIM4_CH2 – Reserved for future use.

Channel 3 = PWM Generation CH3 = PB8 = TIM4_CH3 – Reserved for future use.

Channel 4 = PWM Generation CH4 = PB9 = TIM4_CH4 – Reserved for future use.

PSC = 39

Counter Period = 4999

2000 counts = 1 msec. (ESC has a 1ms to 2ms range)

The following test data was taken:

| Counts | RPM |
| --- | --- |
| 2200 | 2722 |
| 2250 | 4253 |
| 2300 | 5780 |
| 2350 | 6717 |
| 2400 | 7461 |
| 2450 | 8045 |
| 2500 | 8573 |
| 2550 | 8891 |
| 2600 | 9294 |
| 2650 | 9375 |
| 2700 | 9801 |
| 2750 | 9899 |
| 2800 | 10052 |
| 2850 | 10171 |
| 2900 | 10219 |

RPM Sensor to NUCLEO-L476RG

| RPM Sensor | NUCLEO-L476RG |
| --- | --- |
| Red | ESC 5V |
| Black | ESC GND |
| White | PA8 – TIM1_CH1 |

Channel 1 = Input Capture Direct Mode

Channel 2 = Input Capture Indirect Mode

PSC = 79

“Timer 1” is used. The pulse is scaled for 1E-6 sec/count.

It was found that the motor spins at 2567 RPM when the pulse was 1.68 msec. The scale factor was found to be 4.31256 rpm-sec. So to compute the RPM take 4.31256 rpm-sec/0.00168 sec = 2567 RPM. A DT-2234C+ Digital Tachometer was used to verify the scaling of the RPM sensor.

This is the test system used. (test_system.jpg)

motor1.jpg – Note the motor label.

motor2.jpg – Note the motor label.

esc_rpm_sensor.jpg – This is the ESC and rpm sensor used.

battery.jpg – This is the 11.1 Vdc battery used. It is rated for 2200mAh.

rpm_sensor_manual.jpg

esc_manual01.jpg

esc_manual02.jpg

RPM Controller –

RPM - Control Loop