07/16/2022 – Saturday – Using a HP-7090A Plotter to Characterize a Relay

This see through relay has the following markings:

OEG SRE-202D

C0123

LR74323

It coil was measured to be 160 ohms.

The HP-7090A plotter has three recording channels. It will collect 1000 points per channel. Its update rate depends on the total time selected. It fastest time is 0.03 seconds. Therefore, the update time is 0.00003 sec which provides an update rate up to 33,333 Hz.

Rather than using an oscilloscope, this tools is more convenient for testing out slower devices such as a relay. This is because it gives you a nice paper copy that you can used for your documentation and you can write on it.

NO = Normally Open Contact

NC = Normally Closed Contact

Plot 1 – Energize the relay with no diode. (Current)

Coil Current = 72.9 mA

NO Switch time = 8.5 msec.

Plot 2 – De-Energize the relay with no diode. (Current)

Back EMF High

NO Switch time = 3.15 msec.

Plot 3 – Energize the relay with no diode. (Contacts)

NO Switch Time = 7.7 msec

NC Switch Time = 5 msec

Dead Time = 2.7 msec

Break before Make

Plot 4 – De-Energize the relay with 1N4001 diode. (Current)

NO Switch time = 11.9 msec. (Slower with Diode)

Plot 5 – Energize the relay with 1N4001 diode. (Current)

NO Switch Time = 7.9 msec. (Not any slower with the diode.)

Plot 6 – Energize the relay with 1N4001 diode. (Contacts)

NO Switch Time = 7.9 msec

NC Switch Time = 5.6 msec

Dead Time = 2.3 msec

Break before Make

(No difference with Diode)

Plot 7 – De-Energize the relay with 1N4001 diode. (Contacts)

NO Switch time = 12.35 msec. (Slower with Diode)

Lots of Bounce Identified on NC Contact from Diode.

Fuzzy Dead Time

Relay Under Test

Knife-Switch

Battery for Contact Testing

1-Ohm Shunt Resistor for measuring Current

1N4001 Diode

Plotter Results

HP-6038A Power Supply

HP-7090A-Plotter

HP-7090A-Plotter-Left-Side

Test Set-Up

Test Set-Up